Revised Draft

Ozone Transport Commission Electric Generating Unit Emission Inventory Analysis

September 18, 2014

Executive Summary

Ozoe oo

Introduction

The Ozone Transport Commission (OTC) Stationary and Area Source Committee (SAS) was directed to identify the largest individual and groupings of emitters of nitrogen oxides (NOx) and volatile organic compounds (VOCs) located in an OTC state or an area that contributes to ozone levels in an OTC state.  SAS was specifically directed to:

(1) examine individual sources and categories of sources with high short-term emissions of NOx or VOCs; and

(2) review electric generating unit (EGU) NOx emission rates to adjust long- term and short-term expectations for emissions reductions; and

(3) develop  state-by-state EGU NOx emission rates achievable considering reasonably available controls.

SAS was additionally directed  to “Evaluate OTR, super regional, and national goals and means to reduce the emissions in a technical and cost effective manner from the identified units and groupings.  The Committee should develop additional strategies, if necessary to reduce peak emissions from these units.”

SAS fomerd the Largest Contributor Workgroup to fullfill the SAS Charge. An EGU Subgroup (Subgroup) was formed to specifally evaluate EGU emissions and evalauet the task listed above.  This Whitepaper details the analysis conducted to date. The Subgroup, with the assistance of SAS and the OTC Modeling Committee, will perform additional cost-effectiveness and air quality impact analyses as necessary.

Project Scope

The Subgroup was directed to identify the largest individual and groupings of emitters of NOx within and outside the Ozone Transport Region (OTR) by reviewing recent state, regional, and national emissions data. The Subgroup was additionally directed to evaluate the feasibility of reducing peak emissions and to establish reasonably available control technology-based emissions rate limits.   Review of the data was completed to:

- determine the highest short term emission sources regardless of total emissions; and

- evaluate NOx emission rates for EGUs considering multiple factors and

- develop strategies for adjusting short term and long term expectations for emission rates for EGUs considering age, controls in use and fuel type on a unit by unit basis.

The results of these analyses are a potential state-by-state EGU ozone season NOx budget and short term ozone season NOx emission rates considering RACT and allowing for adjustments based on state specific knowledge on a case by case basis.  The results of these data analyses will be used as inputs to the Eastern Regional Technical Advisory Committee (ERTAC) modeling tool.

The Subgroup, with the assistance of SAS and the OTC Modeling Committee will perfrom additional cost-effectiveness and air quality impact analyses as necessary. The results of these analyses may be used to make recommendations to the United States Environmental Protection Agency (EPA) for future EGU regulations.

Project Results

Operation of Emissions Controls

The analysis of the Top 25 Ozone Season NOx & SO2 Emitters in the OTC Modeling Domain for 2011and 2012 and 2013 demonstrates that some EGUs equipped with NOx emissions controls are emitting NOx at rates and in  amounts equal to the pre-installation of post-combustion NOx controls.6  In 2012 approximately 35% of the coal-fired units equipped with post combustion NOx controls had average ozone season NOx emission rates at least 50% higher than its lowest ozone season NOx emission rate between 2003 and 2012. This data suggests that some EGU’s are either not operating or limiting the operation of their existing air pollution control devices.

Analysis 1 Ozone Season NOx Controls and EGU Retirements

The results of NOx control installation and the separate analysis on the potential impact of EGU retirements on ozone season NOx emissions will vary from state to state.  Some states anticipate no coal-fueled EGU retirements while other states anticipate a significant amount of coal-fueled EGU retirements. Analysis 1 results demonstrate that significant NOx reductions can be achieved through the application of reasonably available controls, beyond what is achieved through retirements and fuel switching from coal to natural gas.

Analysis 2 Short Term- Hourly EGU NOx emissions during a high ozone period

The results of the State of Delaware June 21-22, 2012 hourly EGU NOx emissions and hourly NOx emission rates demonstrate EGU NOx emissions varied on an hourly basis with maximum emissions occurring during hour 16 on June 20, 2012.  NOx emission rates from all types of coal-fired EGU peaked during this period.   The review of the related data also indicated:

- Many EGUs cycled on and off line during the period to meet the grid’s electric demand, including a number of coal-fired EGUs;

- While the period experienced an air quality episode, many EGUs remained off line throughout the period, raising concerns  if the electric demand was higher thereby causing additional EGUs to be brought on line;

- During hour 16, states subject to the CAIR ozone season NOx program, coal- and natural gas-fired EGUs were responsible for the greatest heat input, with coal-fired EGU contributing approximately 79% and natural gas-fired EGUs contributing approximately 15% of the total NOx mass emissions.

Analysis 2 results demonstrate that hourly NOx emission rates from a number of EGUs were greater during the period studied than expected from units with pollution controls installed.

Analysis 3 Short Term Daily EGU NOx Emissions

The results of the 2011 daily EGU NOx emissions analyses demonstrate that daily EGU NOx emissions increased with the ambient temperature, with the highest daily EGU NOx emissions occurring on days with the highest daily temperatures.  A large amount  of EGU NOx emitted on high energy demand days (HEDD) in the OTR and Lake Michigan Air Directors Consortium (LADCO) region during the 2011 ozone season were from coal-fired units.  NOx emissions from EGUs firing other fuels (e.g., diesel, residual oil, natural gas) were very small in the LADCO region, while their contribution was significant in the OTR, especially on HEDD.

Analysis 4 “Coal SCR Scorecard”

The results of the “Coal SCR Scorecard” analysis illustrates the relative performance of SCR coal units in listed states. The variations illustrate differing state regulations with respect to NOx emissions. Analysis results indicate some EGUs are either not operating or limiting the operation of their pollution control devices.

Analysis 5- Recommendation for Modeling of Short Term NOx Emission Limits

The NOx emission rates for some EGUs in recent ozone seasons were significantly higher than the NOx emission rate demonstrated by those EGUs in previous years and those expected from units with installed NOx controls.

A potential solution to the air quality problems caused by sources not operating or limiting the  operation of their emission controls  is the establishment of short term NOx emission rate limits for EGUs based on state reported short term NOx emission rates and reflective of control practices using reasonably available applicable NOx emissions controls.

The proposed short term NOx emission rates shown below are reflective of the reasonable application of NOx controls and representative of the capabilities of layered combustion controls or post-combustion controls in retrofit installations.  The proposed short term NOx emission rate limits account for varied EGU configurations and fuel differences. The proposed short term NOx emission rate limits include averaging periods that are necessary to support attainment and maintenance of short term air quality standards, and are expected to be sustainable over a long period provided operators follow good operating and maintenance practices.

If the proposed short term NOx emission rate limits are adopted by regulatory bodies (state rules, regional MOUs, potential federal rule), in addition to an expectation of general air quality improvement, the reductions would be especially effective during periods of high electric demand which often correspond to air quality episodes.  The short term NOx emission rate limits would therefore be expected to help reduce the frequency and magnitude of air quality episodes in the OTR.
The proposed short term NOx emission rate limits are included in the following table:

| Unit Type | Heat Input Capacity (MMBtu/hr) | Configuration | NOx Limit (lb/MMBtu) | Averaging Period |
| --- | --- | --- | --- | --- |
| Boiler - Solid Fuel | HI ≥ 1000 | Arch | 0.125 | 24-hours |
|  |  | Cell | 0.125 | 24-hours |
|  |  | CFB | 0.125 | 24-hours |
|  |  | Cyclone | 0.150* | 24-hours |
|  |  | Stoker | 0.150 | 24-hours |
|  |  | Tangential | 0.125 | 24-hours |
|  |  | Wall | 0.125 | 24-hours |
| Boiler - Solid Fuel | HI < 1000 | Arch | 0.150 | 24-hours |
|  |  | Cell | 0.150 | 24-hours |
|  |  | CFB | 0.125 | 24-hours |
|  |  | Cyclone | 0.150* | 24-hours |
|  |  | Stoker | 0.150 | 24-hours |
|  |  | Tangential | 0.150 | 24-hours |
|  |  | Wall | 0.150 | 24-hours |
| Boiler - Gas Fuel | All | All | 0.125 | 24-hours |
| Boiler - Distillate Oil Fuel | All | All | 0.125 | 24-hours |
| Boiler - Residual Oil Fuel | All | All | 0.150 | 24-hours |
| Combustion Turbine - Gas Fuel | All | Simple Cycle | 25 ppmvd@15%O2* | 1-hour |
|  |  |  | 0.10 lb/MMBtu | 1-hour |
|  |  |  | 1.0 lb./MWh** | 1-hour |
|  |  | Combined Cycle | 25 ppmvd@15%O2* | 1-hour |
|  |  |  | 0.10 lb/MMBtu | 1-hour |
|  |  |  | 0.75 lb/MWh** | 1-hour |
| Combustion Turbine - Oil Fuel | All | Simple Cycle | 42 ppmvd@15%O2* | 1-hour |
|  |  |  | 0.16 lb/MMBtu | 1-hour |
|  |  |  | 1.6 lb/MWh** | 1-hour |
|  |  | Combined Cycle | 42 ppmvd@15%O2* | 1-hour |
|  |  |  | 0.16 lb/MMBtu | 1-hour |
|  |  |  | 1.2 lb/MWh** | 1-hour |

* Some state rules also include provisions for: alternative emission limits NOx RACT orders with alternative NOx RACT emission limits, or the implementation of specific types of NOx control technologies.  Similar alternative compliance means may be necessary for some existing units that may not be able to achieve these NOx rate limits with NOx emission controls representative of RACT.

**lb/MWh emission rates calculated using an efficiency of 35% for simple cycle CTs and 46% for combined cycle CTs [lb/MWh = lb/MMBtu * 3.413 / efficiency]

Ozone Transport Commission Electric Generating Unit Emission Inventory Analysis

- Introduction

The Ozone Transport Commission (OTC) Stationary and Area Source Committee (SAS) was directed to identify the largest individual and groupings of emitters of nitrogen oxides (NOx) and volatile organic compounds (VOCs) located in an OTC state or an area that contributes to ozone levels in an OTC state.  SAS was specifically directed to:

(1) examine individual sources and categories of sources with high short-term emissions of NOx or VOCs;

(2) review electric generating unit (EGU) NOx emission rates to adjust long- term and short-term expectations for emissions reductions; and

(3) develop state-by-state EGU NOx emission rates achievable considering reasonably available controls.

SAS was additionally directed  to “Evaluate OTR, super regional, and national goals and means to reduce the emissions in a technical and cost effective manner from the identified units and groupings.  The Committee should develop additional strategies, if necessary to reduce peak emissions from these units.”

SAS formed the Largest Contributor Workgroup to fullfill the SAS Charge. An EGU Subgroup (Subgroup) was formed to specifically evaluate EGU emissions and evaluate the tasks listed above.  This Whitepaper details the analyses conducted to date. The Subgroup, with the assistance of SAS and the OTC Modeling Committee, will perform additional cost-effectiveness and air quality impact analyses as necessary.

- Project Scope

The Subgroup was directed to identify the largest individual and groupings of emitters of NOx within and outside the Ozone Transport Region (OTR) by reviewing recent state, regional, and national emissions data. The Subgroup was additionally directed to evaluate the feasibility of reducing peak emissions and to establish reasonably available control technology-based emissions rate limits. Review of the data was completed to:

- determine the highest short term emission sources regardless of total emissions; and

- evaluate NOx emission rates for EGUs considering multiple factors; and

- develop strategies for adjusting short term and long term expectations for emission rates for EGUs considering age, controls in use and fuel type on a unit by unit basis.

The Subgroup performed five inventory data analyses. The results of two analyses, a potential state-by-state EGU ozone season NOx budget (Analysis 1); and short term ozone season NOx emission rates considering RACT and allowing for adjustments based on state specific knowledge on a case by case basis (Analysis 5) will be used as inputs to the Eastern Regional Technical Advisory Committee (ERTAC) modeling tool to demonstrate the effect that these recommendations may have on air quality if adopted.

The Subgroup, with the assistance of SAS and the OTC Modeling Committee will perform additional cost-effectiveness and air quality impact analyses as necessary. The results of these analyses may be used to make recommendations to the United States Environmental Protection Agency (EPA) for future EGU regulations.

- Project Criteria

The scope of this inventory analysis is as follows:

- Years:  The years 2011 and 2012 were selected.  Data from the EPA’s Clean Air Markets Division (CAMD) was available for both of these years.   CAMD data from additional years was reviewed in order to fully evaluate the 2011 and 2012 CAMD data.  CAMD data was supplemented with data from other sources (e.g., United States Energy Information Administration (EIA), etc.) and state inventory data where appropriate and as needed. The year 2011 was selected as the baseline year and also used as the primary year of data collection for the state level ozone season NOx mass emissions evaluation and state level ozone season NOx emission rate evaluation.

- Geographic Area:  This analysis was performed for all states in the OTR: Connecticut, Delaware, District of Columbia, Maine, Maryland, Massachusetts, New Hampshire, New Jersey, New York, Pennsylvania, Rhode Island, Vermont, and Virginia.  This analysis was also performed to the extent of available data for all Clean Air Interstate Rule (CAIR) states, all states identified in the Cross-State Air Pollution Rule (CSAPR), and all states included in the current OTC Modeling domain.

- Inventory Sector:  This analysis was performed for all EGUs included in EPA’s CAMD database for the following EPA programs: Acid Rain (ARP), CAIR, CSAPR, and NOx State Implementation Plan (SIP) Call program, where applicable.  Other data sources were reviewed where necessary to supplement EPA’s CAMD data.

For the purposes of the state-by-state EGU ozone season NOx budget analyses only EGUs with capacities of 25 Megawatts (MW) or greater found in EPA’s CAMD database were included. EGU nameplate rating data was obtained from the EIA database as needed.

For the purposes of the daily ozone season NOx emission rate analyses all units reporting to EPA’s CAMD database were included.

- Pollutant considered: Nitrogen Oxides (NOx) was the air pollutant considered.

- Technical Approach

Unit-level Criteria for NOx emissions

The 2011 and 2012 unit level NOx emissions (mass and rate) were obtained from CAMD for ARP, CAIR, and CSAPR reported units.  The following Excel spreadsheets were created and summarized by state in each spreadsheet:

- 2011 Ozone Season NOx

- 2011 High Ozone Episode NOx (hourly and daily, as available)

- 2012 Ozone Season NOx

- 2012 High Ozone Episode NOx (hourly and daily, as available)

Unit-level data elements include:

- State name

- Facility name

- Facility ID

- Unit ID

- NOx emissions (tons)

- NOx Rate (lb/mmBtu) reported

- NOx Rate (lb/mmBtu) calculated

- NOx Rate (lb/MWhr) calculated

- Heat Input (mmBtu)

- Operating Time (hours)

- Number of months reported

- Source Category

- Unit Type

- Fuel Type

- Age of Unit

- Capacity factor

- NOx Controls

- Top 25 Ozone Season NOx Emitters in the OTC Modeling Domain

The Subgroup identified the Top 25 Ozone Season NOx Emitters in the OTC Modeling Domain for 2011, 2012 and 2013. Criteria for inclusion in the list was the mass of NOx emitted during the ozone season, the NOx emission rate was included as additional information.

Top 25 NOx Emitters 2011 Ozone Season

| STATE | Facility Name | Facility ID | UNIT ID | Avg. NOx Rate (lb/MMBtu) | NOx (Tons) |
| --- | --- | --- | --- | --- | --- |
| IN | Rockport | 6166 | MB2 | 0.243 | 5,339 |
| PA | Keystone | 3136 | 2 | 0.363 | 5,044 |
| PA | Keystone | 3136 | 1 | 0.371 | 4,855 |
| PA | Hatfield’s Ferry Power Station | 3179 | 1 | 0.492 | 4,288 |
| PA | Conemaugh | 3118 | 2 | 0.317 | 4,086 |
| PA | Hatfield’s Ferry Power Station | 3179 | 2 | 0.474 | 3,984 |
| AR | White Bluff | 6009 | 1 | 0.275 | 3,956 |
| PA | Conemaugh | 3188 | 1 | 0.341 | 3,890 |
| PA | Brunner Island | 3140 | 3 | 0.376 | 3,834 |
| AR | White Bluff | 6009 | 2 | 0.279 | 3,794 |
| IN | Rockport | 6166 | MB1 | 0.237 | 3,616 |
| OH | W H Zimmer Generation Station | 6019 | 1 | 0.218 | 3,559 |
| AR | Independence | 6641 | 1 | 0.259 | 3,302 |
| PA | Montour | 3149 | 1 | 0.332 | 3,298 |
| PA | Montour | 3149 | 2 | 0.315 | 3,132 |
| PA | Hatfield’s Ferry Power Station | 3179 | 3 | 0.432 | 2,848 |
| MI | Monroe | 1733 | 2 | 0.285 | 2,811 |
| GA | Harlee Branch | 709 | 4 | 0.407 | 2,806 |
| WV | Fort Martin Power Station | 3943 | 1 | 0.351 | 2,660 |
| NY | Lafarge Building Material, Inc. | 880044 | 41000 |  | 2,647 |
| AR | Independence | 6641 | 2 | 0.227 | 2,463 |
| KY | Paradise | 1378 | 3 | 0.386 | 2,431 |
| NY | Somerset Operating Company (kintigh) | 6082 | 1 | 0.296 | 2,347 |
| OH | Avon Lake Power Plant | 2836 | 12 | 0.400 | 2,328 |
| OH | EastLake | 2837 | 5 | 0.262 | 2,323 |
| * Red Text Indicates Units Scheduled to Retire |  |  |  |  |  |
| * Blue Text indicates Units with Future Controls Planned |  |  |  |  |  |
| * Pink Text indicates Units with Installed Pollution Controls |  |  |  |  |  |
| * LaFarge Building Material, Inc, is not an EGU |  |  |  |  |  |
| *Conemaugh has installed controls on Unit 1 and 2 |  |  |  |  |  |
| * Hatfield's Ferry closed as of October 2013 |  |  |  |  |  |

Top 25 NOx Emitters 2012 Ozone Season

| STATE | Facility Name | Facility ID | UNIT ID | Avg. NOx Rate (lb/MMBtu) | NOx (Tons) |
| --- | --- | --- | --- | --- | --- |
| MO | New Madrid Power Plant | 2167 | 1 | 0.627 | 5,786 |
| IN | Rockport | 6166 | MB1 | 0.221 | 5,001 |
| PA | Keystone | 3136 | 1 | 0.365 | 4,661 |
| IN | Rockport | 6166 | MB2 | 0.224 | 4,215 |
| MO | New Madrid Power Plant | 2167 | 2 | 0.505 | 4,134 |
| PA | Conemaugh | 3118 | 1 | 0.320 | 3,909 |
| PA | Montour | 3149 | 2 | 0.414 | 3,794 |
| PA | Conemaugh | 3118 | 2 | 0.300 | 3,789 |
| PA | Keystone | 3136 | 2 | 0.343 | 3,774 |
| PA | Hatfield’s Ferry Power Station | 3179 | 3 | 0.509 | 3,677 |
| PA | Hatfield’s Ferry Power Station | 3179 | 1 | 0.486 | 3,601 |
| PA | Hatfield’s Ferry Power Station | 3179 | 2 | 0.520 | 3,589 |
| PA | Montour | 3149 | 1 | 0.402 | 3,543 |
| AR | White Bluff | 6009 | 1 | 0.278 | 3,504 |
| AR | White Bluff | 6009 | 2 | 0.246 | 3,383 |
| MO | Thomas Hill Energy Center | 2168 | MB2 | 0.684 | 3,236 |
| AR | Independence | 6641 | 2 | 0.205 | 2,816 |
| WV | Fort Martin Power Station | 3943 | 1 | 0.319 | 2,730 |
| AL | E C Gaston | 26 | 5 | 0.203 | 2,656 |
| WV | Harrison Power Station | 3944 | 3 | 0.308 | 2,628 |
| PA | Brunner Island | 3140 | 3 | 0.346 | 2,601 |
| WV | Harrison Power Station | 3944 | 1 | 0.313 | 2,569 |
| MI | Monroe | 1733 | 2 | 0.259 | 2,536 |
| MI | Monroe | 1733 | 1 | 0.247 | 2,517 |
| OH | Killen Station | 6031 | 2 | 0.351 | 2,426 |
| * Red Text Indicates Units Scheduled to Retire |  |  |  |  |  |
| * Blue Text indicates Units with Future Controls Planned |  |  |  |  |  |
| * Pink Text indicates Units with Installed Pollution Controls |  |  |  |  |  |
| * Conemaugh has installed controls on Unit 1 and 2 |  |  |  |  |  |
| * Hatfield's Ferry closed as of October 2013 |  |  |  |  |  |

Top 25 NOx Emitters 2013 Ozone Season

| STATE | Facility Name | Facility ID | UNIT ID | Avg. NOx Rate (lb/MMBtu) | NOx (Tons) |
| --- | --- | --- | --- | --- | --- |
| MO | New Madrid Power Plant | 2167 | 2 | 0.499 | 4,328 |
| OH | W H Zimmer Generating Station | 6019 | 1 | 0.219 | 4,261 |
| AR | White Bluff | 6009 | 2 | 0.283 | 4,193 |
| MO | New Madrid Power Plant | 2167 | 1 | 0.609 | 4,126 |
| AR | White Bluff | 6009 | 1 | 0.291 | 4,096 |
| PA | Conemaugh | 3118 | 1 | 0.318 | 4,095 |
| IN | Rockport | 6166 | MB1 | 0.213 | 3,997 |
| PA | Hatfield’s Ferry Power Station | 3179 | 2 | 0.491 | 3,876 |
| PA | Hatfield’s Ferry Power Station | 3179 | 1 | 0.471 | 3,712 |
| PA | Conemaugh | 3118 | 2 | 0.321 | 3,605 |
| PA | Hatfield’s Ferry Power Station | 3179 | 3 | 0.444 | 3,365 |
| MI | Monroe | 1773 | 2 | 0.329 | 3,308 |
| PA | Homer City | 3122 | 3 | 0.332 | 3,235 |
| IN | Rockport | 6166 | MB2 | 0.213 | 3,217 |
| MI | Monroe | 1733 | 1 | 0.315 | 3,062 |
| PA | Montour | 3149 | 2 | 0.424 | 3,050 |
| OH | Killen Station | 6031 | 2 | 0.304 | 2,951 |
| WV | Fort Martin Power Station | 3943 | 1 | 0.358 | 2,905 |
| WV | Harrison Power Station | 3944 | 3 | 0.301 | 2,874 |
| PA | Montour | 3149 | 1 | 0.398 | 2,864 |
| WV | Harrison Power Station | 3944 | 1 | 0.280 | 2,817 |
| AR | Independence | 6641 | 2 | 0.209 | 2,807 |
| NC | Marshall | 2727 | 4 | 0.341 | 2,639 |
| PA | Homer City | 3122 | 2 | 0.328 | 2,552 |
| OH | Conesville | 2840 | 5 | 0.479 | 2,530 |
| * Red Text Indicates Units Scheduled to Retire |  |  |  |  |  |
| * Blue Text indicates Units with Future Controls Planned |  |  |  |  |  |
| * Pink Text indicates Units with Installed Pollution Controls |  |  |  |  |  |
| *Conemaugh has installed controls on Unit 1 and 2 |  |  |  |  |  |
| * Hatfield's Ferry closed as of October 2013 |  |  |  |  |  |

The Top 25 NOx emitters for the 2011, 2012 and 2013 ozone seasons indicate that while there are units that appear during all three years, there is also a  high degree of variation during the three years. The variation may be attributed to changes in fuel prices affecting economic dispatch, maintenance outages, electric demand, operation and/or effectiveness of installed NOx, controls, etc.  The Subgroup expects that, due to the factors listed above and anticipated pollution controls, the list of Top 25 NOx emitters will continue to change year to year.

The EGUs identified on the lists include EGUs equipped with combustion NOx controls, post-combustion NOx controls, and combinations of both types of NOx controls. The EGUs identified on the list have commonalities, specifically, they are all relatively large coal-fired steam units with average ozone season NOx emission rates that do not reflect the NOx reduction capabilities of modern, layered combustion controls or post-combustion NOx controls. While the EGUs identified in this section are located in the OTC modeling domain, it is indicative of the largest ozone season NOx emitting EGUs on a national fleet basis.

The lists demonstrate that some EGUs equipped with NOx emissions controls are emitting NOx at rates and amounts equal to the pre-installation of post-combustion NOx controls. In 2012 approximately 35% of the coal-fired units equipped with post combustion NOx controls had average ozone season NOx emission rates at least 50% higher than its lowest ozone season NOx emission rate between 2003 and 2012. This data suggests that some EGU’s are not operating, or limiting the operation of their controls.

- Analyses and Results

- Analysis 1: Ozone Season NOx Emission Controls and Unit Retirements

Analysis

Data from the EPA’s CAMD Air Markets Program Data (AMPD) database (i.e., ARP, CAIR, and CSAPR program data) and information from EIA were used to examine reasonably cost‐effective post combustion EGU control technologies and to determine fleet‐wide average NOx emission rates for fossil fuel‐fired EGUs.

EGU data was used to identify existing controls and to determine average 2011 actual ozone season NOx emission rates.  By applying an enhanced EGU control strategy, a revised 2011 ozone season NOx mass emissions were calculated.  The calculation process included the following:

General:

- The year 2011 was selected as the base year for determining the baseline ozone season EGU fleet, EGU ozone season NOx mass emissions, and EGU ozone season heat input.

- The fleet of EGUs was identified in the CAMD AMPD database as electric utility or small power producers with nameplate capacity ≥ 25 MW, excluding units identified as co-generation or any industrial, commercial, or process unit.

- For existing EGUs with post-combustion NOx controls, each EGU’s NOx emissions rate (lb/MMBTU) was obtained from CAMD AMPD data and the lowest ozone season average NOx emissions rate between 2003 and 2012, inclusive, was selected.  Each EGU’s capacity factor was calculated from the CAMD AMPD data.

- The 2012 ozone season values were included in this analysis as it was the most recent ozone season average NOx emission rate available and to potentially provide credit to an individual EGU for NOx controls and/or NOx emission rate reductions that have already been incorporated on that EGU.

- For each EGU, estimated ozone season NOx emissions were calculated as the product of the actual 2011 NOx mass emissions and the ratio of the estimated ozone season NOx emissions rate after application of controls and the actual 2011 ozone season average NOx emissions rate as follows:

Estimated Ozone Season =

(Actual 2011 OS NOx Mass Emission) *(Estimated NOx Emission Rate After Control/Actual 2011 OS NOx Emission Rate)

- Coal-Fueled EGUs:

A coal-fueled EGU was any EGU identified in the CAMD AMPD database that included coal or coal-refuse as a primary fuel or secondary fuel.

Coal-fueled EGUs of any size that were identified in the CAMD AMPD as having incorporated Selective Catalytic Reduction Technology (SCR), the estimated ozone season NOx emissions rate was the lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012.

If the lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012 was 0.06 lb/MMBTU or less, 0.06 lb/MMBTU was used as the estimated ozone season NOx emissions rate regardless of the NOx controls installation indicated in the AMPD.

Coal-fueled EGUs with a heat input rating of 2000 MMBTU/hr, or greater:

1) Coal-fueled EGUs identified in the AMPD as incorporating Selective Non-Catalytic Reduction Technology (SNCR) with a lowest demonstrated ozone season NOx emissions rate between the years 2003 and2012 greater than 0.06 lb/MMBTU, installation of SCR was assumed and the NOx emissions rate was estimated as 50% of the lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012.  The floor NOx emissions rate for this estimation was 0.06 lb/MMBTU.

2) Coal-fueled EGUs identified in the AMPD as incorporating neither SNCR nor SCR with a lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012 greater than 0.06 lb/MMBTU, installation of SCR was assumed and the NOx emissions rate was estimated as 10% of the lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012.  The floor NOx emissions rate for this estimation was 0.06 lb/MMBTU.

Coal-fueled EGUs with a heat input rating of 1000 MMBTU/hr, or greater, but less than 2000 MMBTU/hr:

- Coal-fueled EGUs identified in the AMPD as incorporating SNCR and with a 2011 ozone season heat input capacity factor less than 40% of the total capacity, the estimated ozone season NOx emissions rate was the lowest demonstrated ozone season NOx emissions between the years 2003 and 2012.

- Coal-fueled EGUs  identified in the AMPD as incorporating SNCR with a lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012 greater than 0.06 lb/MMBTU and a 2011 ozone season heat input capacity factor 40% or greater of the total capacity, installation of SCR was assumed.  The NOx emissions rate was estimated as 50% of the lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012.  The floor NOx emissions rate for this estimation was 0.06 lb/MMBTU.

- Coal-fueled EGUs identified in the AMPD as incorporating neither SCR nor SNCR, with a lowest demonstrated ozone season NOx emissions rate between the  years 2003 and 2012 greater than 0.06 lb/MMBTU and the 2011 ozone season heat input capacity factor 40% or greater of the total capacity , installation of SCR was assumed. The NOx emissions rate was estimated as 10% of the lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012.  The floor NOx emissions rate for this estimation was 0.06 lb/MMBTU.

- Coal-fueled EGUs identified in the AMPD as incorporating neither SCR nor SNCR, with a lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012 greater than 0.06 lb/MMBTU and the 2011 ozone season heat input capacity factor less than 40% of the total capacity, installation of SNCR was assumed. The NOx emissions rate was estimated as 60% of the lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012.  The floor NOx emissions rate for this estimation was 0.06 lb/MMBTU.

Coal-fueled EGUs with a heat input rating of less than 1000 MMBTU/hr:

- Coal-fueled EGUs identified in the AMPD as incorporating SCR or SNCR, the estimated ozone season NOx emissions rate used was the lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012.

2)  Coal-fueled EGUs identified in the AMPD as incorporating neither SNCR nor SCR, installation of SNCR was assumed.  The NOx emissions rate was estimated as 60% of the lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012.  The floor NOx emissions rate for this estimation was 0.06 lb/MMBTU.

- Non-Coal Fueled Boilers Serving EGUs

Non-coal fueled boilers serving EGUs were those EGU boilers identified in the AMPD as not including coal or coal-refuse as a primary or secondary fuel.

If the non-coal fueled EGU boiler’s lowest demonstrated ozone season NOx emissions rate between the years 2003 and2012 was less than 0.1 lb/MMBTU, 0.1 lb/MMBTU was the estimated ozone season NOx emissions rate regardless of the NOx controls installation indicated in the AMPD.

Non-coal-fueled EGU with a heat input rating of 2000 MMBtu/hr. or greater:

- Non-coal fueled EGU boilers identified in the AMPD as incorporating SCR or SNCR, the NOx emission rate was the lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012.

- Non-coal fueled EGU boilers identified in the AMPD as incorporating neither SCR nor SNCR, with a lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012 greater than 0.1 lb/MMBTU, installation of SCR was assumed.  The NOx emissions rate was estimated as 20% of the lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012.  The floor NOx emissions rate for this estimation was 0.06 lb/MMBTU.

Non-coal fueled EGU boilers with a heat input rating of 1000 MMBTU/hr, or greater, but less than 2000 MMBTU/hr:

1)   Non-coal fueled EGU boilers identified in the AMPD as incorporating SCR; the

estimated ozone season NOx emissions rate was the lowest demonstrated

ozone season NOx emissions rate between the years 2003 and 2012.

2)  Non-coal fueled EGU boilers identified in the AMPD as incorporating SNCR with

a 2011 ozone season heat input capacity factor less than 40% of the total capacity, the estimated ozone season NOx emissions rate was the lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012.

- Non-coal fueled EGU boilers identified in the AMPD as incorporating SNCR, with a 2011 ozone heat input capacity factor 40% or greater of the total capacity, installation of SCR was assumed.  The NOx emission rate was estimated at 70% of the lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012.  The floor NOx emission rate for this estimation was 0.06 lb/MMBTU.

4)   Non-coal fueled EGU boilers identified in the AMPD as incorporating neither     SCR nor  SNCR,  with a  lowest demonstrated emissions rate between the years 2003  and 2012 greater than 0.1 lb/MMBTU and the 2011 ozone season heat input  capacity factor 40% or greater of the total capacity, installation of SCR was      assumed. The NOx emissions rate was estimated as 20% of the lowest

demonstrated ozone season average NOx emissions rate between the years 2003 and 2012.  The floor NOx emissions rate for this estimation was 0.06 lb/MMBTU.

5)   Non-coal-fueled EGU boilers identified in the AMPD as incorporating neither  SCR nor SNCR, with a  lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012 greater than 0.06 lb/MMBTU and the 2011 ozone season heat input capacity factor was less than 40% of the total capacity, installation of SNCR was assumed. The NOx emissions rate was estimated as 50% of the lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012.  The floor NOx emissions rate for this estimation was 0.06 lb/MMBTU.

Non-coal-fueled EGUs with a heat input rating of less than 1000 MMBTU/hr:

- Non-coal fueled EGU boilers identified in the AMPD as incorporating SCR or

SNCR, the estimated NOx emission rate was the lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012.

- Non-coal fueled EGU boilers identified in the AMPD as incorporating neither SCR nor SNCR with a  lowest demonstrated ozone season     NOx emissions rate between the years 2003 and 2012  greater than 0.1 lb/MMBTU installation of SNCR was assumed. The NOx emissions rate was estimated as 60% of the lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012. The floor NOx emissions rate for this estimation was 0.06 lb/MMBTU.

- Combined Cycle (CC) and Combustion Turbine (CT) EGUs:

- If the CC or CT EGU’s lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012 was less than 0.1 lb/MMBTU, 0.1 lb/MMBTU was  the estimated ozone season NOx emissions rate regardless of the NOx controls installation indicated in the AMPD.

- If the CC or CT EGU’s lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012 was 0.1 lb/MMBTU or greater, and the EGU was identified in the AMPD as incorporating Dry Low NOx Burner (DLNB) or water injection, then the lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012 was the estimated ozone season NOx emissions rate.

- If the CC or CT EGU’s lowest demonstrated ozone season NOx emissions rate between the years 2003 and 2012 was 0.1 lb/MMBTU or greater, and the EGU was not identified in the AMPD as incorporating DLNB or water injection, installation of water injection was assumed.  The NOx emissions rate was estimated as 60% of the lowest demonstrated ozone season average NOx emissions rate between the years 2003 and 2012.

For CC or CT EGU’s that appear to be utilizing default values and did not indicate incorporation of DLNB, water injection, or SCR, the NOx emissions reductions from those units was estimated as follows:

1)   For a CC or CT unit, a NOx emissions rate estimate was calculated using the non-default average NOx emission rates for CCs or CTs (as appropriate) using the same primary fuel type and same heat input classification.

2)   Using the AMPD reported 2011 heat input for that CC or CT EGU, the “actual” NOx mass emissions was calculated by multiplying the heat input with the above estimated NOx emissions rate.

3)   Assuming installation of water injection and a resulting 40% reduction in NOx emissions rate, the reduction of NOx mass emissions is estimated as 40% of the “actual” NOx mass emissions.

As the above estimates are made on a unit-specific basis, NOx mass caps could be calculated on a regional basis (state specific, CAIR region, etc).  The process outlined above allows for a NOx mass cap calculation representative of the existing EGU fleet and its ability to achieve NOx emissions reductions. It also identified areas where some of the existing regulatory and economic processes have produced some NOx reduction success (such as increased use of well-controlled gas-fueled combined cycle units) and areas where NOx reductions have diminished (such as discontinuing or ineffectively using existing NOx controls on some coal-fired units).

Results

The following graphs show the impact of Analysis 1 NOx controls, and the potential impact of EGU retirements on state level ozone season NOx mass emissions in tons.  The spreadsheets used to create these graphs can be found in Appendix 3.

Estimated Impact of Coal-Fired EGU Retirements and Analysis 1 NOx Controls on Ozone Season EGU NOx Emissions for OTC States (Revised 11.25.2013)

(measured in Tons)

Estimated Impact of Coal-Fired EGU Retirements and Analysis 1 NOx Controls on Ozone Season EGU NOx Emissions for LADCO States

(Revised 11.25.2013) (measured in Tons)

Estimated Impact of Coal-Fired EGU Retirements and Analysis 1 NOx Controls on Ozone Season EGU NOx Emissions for VISTAS States minus Virginia          (Revised 11.25.2013) (measured in Tons)

Estimated Impact of Coal Fired EGU Retirements and Analysis 1 NOx Controls on Ozone Season EGU NOx Emissions Regional Summary

(Revised 11.25.2013) (Measured in Tons)

The following graphs demonstrate the potential impact of Analysis 1 NOx controls and the potential impact of EGU retirements on state level ozone season NOx emission rates in lb.NOx/MMBtu.  The spreadsheets used to create these graphs can be found in Appendix 4.

Estimated Impact of Coal Fired EGU Retirements and Analysis 1 NOx Controls on Ozone Season Fleet Average NOx Emission Rates for OTC States

(Revised 11.25.2013) (Measured in lb/MMBtu)

Estimated Impact of Coal Fired EGU Retirements and Analysis 1 NOx Controls on Ozone Season Fleet Average NOx Emission Rates for LADCO States

(Revised 11.25.2013) (Measured in lb/MMBtu)

Estimated Impact of Coal Fired EGU Retirements and Analysis 1 NOx Controls on Ozone Season Fleet Average NOx Emission Rates for VISTAS States Minus Virginia (Revised 11.25.2013) (Measured in lb/MMBtu)

The results of the NOx control installation analysis and the separate analysis on the potential impact of EGU retirements on ozone season NOx emissions will vary from state to state.  Some states anticipate no coal-fueled EGU retirements while other states anticipate a significant amount of coal-fueled EGU retirements. Analysis 1 results demonstrate that significant NOx reductions can be achieved through the application of reasonably available controls, beyond what is achieved through retirements and fuel switching from coal to natural gas.

- Analysis 2: Short Term- Hourly EGU NOx Emissions during a High Ozone Period

Analysis

The State of Delaware prepared an analysis of hourly EGU NOx emissions and hourly EGU NOx emission rates during a high ozone period in Delaware. The Subgroup prepared a High Energy Demand Day (HEDD) analysis for the OTC Modeling Domain on: Low Emitting Combustion Turbines (LECTs with NOx emissions <0.125 lb/mmBtu), High Emitting Combustion Turbines (HECTs with NOx emissions >0.125 lb/mmBtu) and coal-fired EGUs with and without SCR controls installed during a high ozone period in Delaware & New Jersey.

Results

The results of the State of Delaware hourly EGU NOx emissions and hourly NOx emission rates (June 21-22, 2012) study demonstrate EGU NOx emissions varied on an hourly basis with maximum emissions occurring during hour 16 on June 20, 2012.  NOx emission rates from all types of coal-fired EGU also peaked during this time.  The review of the related data also indicated:

- Many EGUs were cycled on and off line during the period to meet the grid’s electric demand, including a number of coal-fired EGUs;

- While the period experienced an air quality episode, many EGUs remained off line throughout the period, raising concerns if the electric demand was higher thereby causing additional EGUs to be brought on line;

- The NOx emission rates from a number of EGUs were much greater than would be expected relative to the NOx controls reported to be installed on those units;

- During hour 16, for states subject to the CAIR ozone season NOx program, coal- and natural gas-fired EGUs were responsible for the greatest heat input, with coal-fired EGUs contributing approximately 79% and natural gas-fired EGUs contributing approximately 15% of the total NOx mass emissions.

State of Delaware hourly EGU NOx emissions and hourly NOx emission rates

(June 21-22, 2012)

Hourly NOx Rate on 06.20.12 for hour 16- Operating Coal Units for Connecticut, Delaware, Illinois, Indiana, Kentucky, Maryland, Michigan, New Jersey, New York, Ohio, Pennsylvania, Tennessee

Results of Subgroup HEDD analysis for the OTC Modeling Domain on: Low Emitting Combustion Turbines (LECTs with NOx emissions <0.125 lb/mmBtu), High Emitting Combustion Turbines (HECTs with NOx emissions >0.125 lb/mmBtu) and coal-fired EGUs with and without SCR controls are presented in the following graphs.

- Analysis 3: Short Term Daily EGU NOx Emissions

Analysis

An update of a previous analysis done in 2007 for daily NOx emissions by fuel type and maximum daily temperature for EGUs located in the OTR and Lake Michigan Air Directors Consortium (LADCO) states was performed.

The total daily EGU NOx emission for each fuel type was calculated to determine each fuel-type’s contribution to daily regional NOx emissions. The 2011 unit-level EGU NOx emissions data was downloaded for each state from EPA’s AMPD website. The unit-level NOx emissions data was summed by state and fuel type for each ozone-season day (May 1, 2011 through September 30, 2011).  The state-level NOx emissions for the OTC and LADCO states was then totaled by fuel type and the contribution to daily regional NOx emissions of each fuel type was graphed for the OTC and LADCO states. The temperature data was obtained from the National Oceanic and Atmospheric Administration (NOAA) website.

Results

The results demonstrate that daily EGU NOx emissions increased with the ambient temperature, with the highest daily NOx EGU NOx emissions occurring on days with the highest daily temperatures.  In the OTC states, NOx emissions from oil-fired EGU boilers and diesel fuel-fired EGUs also peaked on the days with highest daily temperatures.

The following graphs demonstrate the majority of EGU NOx emitted on HEDD in the OTR and LADCO during the 2011 ozone season were from coal-fired units.  NOx emissions from EGUs firing other fuels (e.g., diesel, residual oil, natural gas) were very small in the LADCO region while their contribution was significant in the OTR, especially on HEDD.

- Analysis 4: “Coal SCR Scorecard

Analysis

A “Coal SCR Scorecard” listing the number of power plants equipped with SCR controls with higher NOx emission rates during the 2011, 2012, and 2013 ozone seasons than previously demonstrated was prepared by the Subgroup.

The scorecards illustrate the relative performance of SCR-equipped coal units in the listed states.  For example, of the 5 SCR-equipped coal plants in Alabama, only 1 operated at an emission rate substantially greater than previously demonstrated or its “optimum” rate in 2011, for a “grade” of  80%. In 2012, another plant in Alabama operated at an emission rate substantially greater than previously demonstrated for a "grade" of 60%, while in 2013 only 4 plants operated, one sub-optimally for a "grade" of 75%.   In Kentucky, 5 plants were sub-optimum in 2011, one of which appeared to not operate its SCR at all for a "grade" of 50%. While in 2012, 1 of the 5 Kentucky plants operations improved for a "grade" of 64%. Whereas, in Maryland and New Jersey, all plants operated their SCRs at optimum levels in 2011 and 2012.  These variations imply differing state regulations with respect to NOx emissions.

Results

The results of the “Coal SCR Scorecard” analysis demonstrate that in several cases power plants equipped with SCR controls had higher NOx emission rates during the 2011, 2012, and 2013 ozone seasons than previously demonstrated.  Analysis results indicate some EGUs are either not operating or limiting the operation of their existing air pollution control devices.

Coal Scorecard- 2011

| State | Plants | Number of Plants in 2011 with NOx Rate > Previously Demonstrated | SCR Off | SCR Less than Optimum | Grade | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| AL | 5 | 1 | 0 | 1 | 80% |  |
| AR | 1 | 0 |  |  | 100% |  |
| DE | 0 | 0 |  |  | 100% |  |
| GA | 4 | 1 | 0 | 1 | 75% |  |
| IA | 1 | 1 |  |  | 0% |  |
| IL | 8 | 1 | 1 |  | 88% |  |
| IN | 10 | 2 | 0 | 2 | 80% |  |
| KY | 10 | 5 | 1 | 4 | 50% |  |
| MA | 2 | 1 |  |  | 50% |  |
| MD | 4 | 0 |  |  | 100% |  |
| MI | 3 | 1 |  |  | 67% |  |
| MN | 1 | 0 |  |  | 100% |  |
| MO | 3 | 2 | 0 | 2 | 33% |  |
| NC | 6 | 1 |  | 1 | 83% |  |
| NH | 1 | 0 |  |  | 0% |  |
| NJ | 4 | 0 |  |  | 100% |  |
| NY | 2 | 1 |  | 1 | 50% | 1 of 4 in 2010 |
| OH | 10 | 3 |  | 3 | 70% |  |
| PA | 5 | 4 | 2 | 2 | 20% |  |
| SC | 5 | 0 |  |  | 100% |  |
| TN | 4 | 0 |  |  | 100% |  |
| VA | 3 | 2 |  | 2 | 33% |  |
| WI | 3 | 0 |  |  | 100% |  |
| WV | 6 | 2 | 0 | 2 | 67% |  |
| Total | 102 | 28 | 3 | 25 |  |  |
| Percentage of Total |  | 27% | 3% | 25% |  |  |

Coal EGU SCR Use Score as of 2011 Ozone Season

Coal Scorecard- 2012

| State | Plants | Number of Plants in 2012 with NOx Rate > Previously Demonstrated | SCR Off | SCR Less than Optimum | Grade | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| AL | 5 | 2 |  | 2 | 60% |  |
| AR | 1 | 0 |  |  | 100% |  |
| DE | 1 | 0 |  |  | 100% |  |
| GA | 4 | 1 | 0 | 1 | 75% |  |
| IA | 1 | 1 |  | 1 | 0% |  |
| IL | 9 | 2 |  | 2 | 78% |  |
| IN | 12 | 3 | 1 | 2 | 75% |  |
| KY | 11 | 4 | 1 | 3 | 64% |  |
| MA | 2 | 1 |  | 1 | 50% |  |
| MD | 4 | 0 |  |  | 100% |  |
| MI | 3 | 0 |  |  | 100% |  |
| MN | 1 | 0 |  |  | 100% |  |
| MO | 3 | 2 | 2 | 0 | 33% |  |
| NC | 6 | 2 | 0 | 2 | 67% |  |
| NH | 1 | 0 | 0 | 0 | 100% |  |
| NJ | 4 | 0 |  |  | 100% |  |
| NY | 2 | 1 |  | 1 | 50% | 1 of 4 in 2010 |
| OH | 10 | 4 | 2 | 2 | 60% |  |
| PA | 5 | 4 | 3 | 1 | 20% |  |
| SC | 5 | 0 |  |  | 100% |  |
| TN | 4 | 0 |  |  | 100% |  |
| VA | 3 | 1 |  | 1 | 67% |  |
| WI | 4 | 0 |  |  | 100% |  |
| WV | 7 | 2 | 1 | 1 | 71% |  |
| Total | 108 | 30 | 10 | 20 |  |  |
| Percentage of Total |  | 28% | 9% | 19% |  |  |

Coal EGU SCR Use Score as of 2012 Ozone Season

Coal Scorecard- 2013

| State | Plants | Number of Plants in 2013 with NOx Rate > Previously Demonstrated | SCR Off | SCR Less than Optimum | Grade | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| AL | 4 | 1 |  | 1 | 75% |  |
| AR | 1 | 0 |  |  | 100% |  |
| DE | 1 | 0 |  |  | 100% |  |
| GA | 4 | 1 | 0 | 1 | 75% |  |
| IA | 1 | 1 |  | 1 | 0% |  |
| IL | 9 | 1 |  | 1 | 89% |  |
| IN | 12 | 3 | 2 | 1 | 75% |  |
| KY | 11 | 4 | 1 | 3 | 64% |  |
| MA | 2 | 1 |  | 1 | 50% |  |
| MD | 4 | 0 |  |  | 100% |  |
| MI | 3 | 1 |  | 1 | 67% |  |
| MN | 1 | 0 |  |  | 100% |  |
| MO | 3 | 2 | 2 |  | 33% |  |
| NC | 6 | 3 |  | 3 | 50% |  |
| NH | 1 | 0 |  |  | 100% |  |
| NJ | 4 | 0 |  |  | 100% |  |
| NY | 2 | 1 |  | 1 | 50% |  |
| OH | 10 | 5 | 2 | 3 | 50% |  |
| PA | 5 | 5 | 3 | 2 | 0% |  |
| SC | 5 | 0 |  |  | 100% |  |
| TN | 4 | 1 |  | 1 | 75% |  |
| VA | 3 | 0 |  | 0 | 100% |  |
| WI | 5 | 0 |  |  | 100% |  |
| WV | 7 | 2 | 1 | 1 | 71% |  |
| Total | 108 | 31 | 11 | 20 |  |  |
| Percentage of Total |  | 29% | 10% | 19% |  |  |

Coal EGU SCR Use Score as of 2013 Ozone Season

- Analysis 5: Recommendation for Modeling of Short Term NOx Emission Limits

Analysis

EGU NOx emission rate data indicates that many of the EGU exhibited average NOx emission rates in excess of what might be expected for EGUs incorporating post-combustion controls. In recent ozone seasons, some EGUs reported to incorporate post-combustion NOx controls have exhibited average NOx emission rates higher than previous ozone season averages.  Application of short term NOx emission rate limits that reflect the capabilities of NOx emissions controls can reduce short term emission rates to rates expected from units with installed post combustion NOx controls.

The short term NOx limits listed in the following tables as “Current Thinking” are not intended to reflect technological edge of NOx control capability, but rather to represent NOx control retrofit capability for much of the EGU industry.  The State rules included in analysis are from Connecticut, Delaware, New Hampshire, New Jersey, New York and Wisconsin.  The averaging times for the EGU boiler NOx limits found in state rules are stated in terms of 24 hr. rolling averages or 24 hr. calendar day averages.   EGU combustion turbine NOx limits found in state rules varied from state to state with some 1 hr avg. limits, some 24 hr avg. limits and some 30 day rolling avg. limits.  The conversion factor used for EGU boilers assumed 0.1 lb/MM Btu ≈ 1.0 lb/MWh.  For simple cycle turbines combusting natural gas fuel it was assumed that 50 ppmvd@15%O2 ≈ 0.1838 lb/MM Btu.  For combined cycle turbines combusting natural gas fuel it was assumed that 42 ppmvd@15%O2 ≈ 0.1544 lb/MMBtu.

Results

A potential solution to the air quality problems caused by sources not operating or limiting the operation of their emission controlling technology is the establishment of short term NOx emission rate limits for EGUs that are based on reported short term NOx emission rates and reflective of good emission control practices using reasonably available NOx emissions controls that are applicable for the particular types of EGUs.

The proposed short term NOx emission rates shown below are felt to be reflective of the capabilities of EGUs with reasonable application of NOx controls when those units are operated in accordance with good emission control practices.  The proposed short term NOx emission rate limits are felt to be representative of the capabilities of layered combustion controls or post-combustion controls in retrofit installations.  In order to ensure that the emission rate reduction capabilities of various EGU configurations and fuel selections are addressed, the proposed short term NOx emission rate limits account for these EGU configurations and fuel differences.

The proposed short term NOx emission rate limits, based on reported short term NOx emission rates, include averaging periods that are felt to be necessary to support attainment and maintenance of short term air quality standards, the proposed short term NOx emission rate limits are expected to be sustainable over a long period of time given good operating and maintenance practices.

If the proposed short term NOx emission rate limits are adopted by regulatory bodies (state rules, regional MOUs, potential federal rule), there would not only be an expectation of general air quality improvement, but it would also be expected to be especially effective during periods of high electric demand which often correspond to air quality episodes.  The short term NOx emission rate limits would therefore be expected to help reduce the frequency and magnitude of those air quality episodes.

The proposed short term NOx emission rate limits are included in the following table:

| Unit Type | Heat Input Capacity (MMBtu/hr) | Configuration | NOx Limit (lb/MMBtu) | Averaging Period |
| --- | --- | --- | --- | --- |
| Boiler - Solid Fuel | HI ≥ 1000 | Arch | 0.125 | 24-hours |
|  |  | Cell | 0.125 | 24-hours |
|  |  | CFB | 0.125 | 24-hours |
|  |  | Cyclone | 0.150* | 24-hours |
|  |  | Stoker | 0.150 | 24-hours |
|  |  | Tangential | 0.125 | 24-hours |
|  |  | Wall | 0.125 | 24-hours |
| Boiler - Solid Fuel | HI < 1000 | Arch | 0.150 | 24-hours |
|  |  | Cell | 0.150 | 24-hours |
|  |  | CFB | 0.125 | 24-hours |
|  |  | Cyclone | 0.150 | 24-hours |
|  |  | Stoker | 0.150 | 24-hours |
|  |  | Tangential | 0.150 | 24-hours |
|  |  | Wall | 0.150 | 24-hours |
| Boiler - Gas Fuel | All | All | 0.125 | 24-hours |
| Boiler - Distillate Oil Fuel | All | All | 0.125 | 24-hours |
| Boiler - Residual Oil Fuel | All | All | 0.150 | 24-hours |
| Combustion Turbine - Gas Fuel | All | Simple Cycle | 25 ppmvd@15%O2* | 1-hour |
|  |  |  | 0.10 lb/MMBtu | 1-hour |
|  |  |  | 1.0 lb./MWh** | 1-hour |
|  |  | Combined Cycle | 25 ppmvd@15%O2* | 1-hour |
|  |  |  | 0.10 lb/MMBtu | 1-hour |
|  |  |  | 0.75 lb/MWh** | 1-hour |
| Combustion Turbine - Oil Fuel | All | Simple Cycle | 42 ppmvd@15%O2* | 1-hour |
|  |  |  | 0.16 lb/MMBtu | 1-hour |
|  |  |  | 1.6 lb/MWh** | 1-hour |
|  |  | Combined Cycle | 42 ppmvd@15%O2* | 1-hour |
|  |  |  | 0.16 lb/MMBtu | 1-hour |
|  |  |  | 1.2 lb/MWh** | 1-hour |

* Some state rules also include provisions for: alternative emission limits, NOx RACT orders with alternative NOx RACT emission limits, or the implementation of specific types of NOx control technologies.  Similar alternative compliance means may be necessary for some existing units that may not be able to achieve these NOx rate limits with NOx emission controls representative of RACT.                                                                                                                      **lb/MWh emission rates calculated using an efficiency of 35% for simple cycle CTs and 46% for combined cycle CTs [lb/MWh = lb/MMBtu * 3.413 / efficiency]

Appendices

1. Ozone Transport Commission charge to the Stationary and Area Source Committee at November 2012 Fall meeting, Attached and available at: http://www.otcair.org/upload/Documents/Formal%20Actions/Charge%20to%20SAS%20Committee.pdf

2. Ozone Transport Commission charge to the Stationary and Area Source Committee at November 2013 Fall meeting available at:

http://www.otcair.org/upload/Documents/Formal%20Actions/Chrg%20to%20SAS%20for%20Reg%20Attainment%20of%20Ozone.pdf

3. Rev 11 25 13 EGU 25 MW MASS Shutdowns 121613 – Estimated NOx Emissions Baseline & CHARTS.xls

4. Rev 11 25 13 EGU 25 MW RATES Shutdowns 121613 – Estimated NOx Emissions Baseline & CHARTS.xls

List of References

1. Statement from the Ozone Transport Commission Requesting the Use  and Operation of Existing Control Devises Installed at Electric Generating Units, June 2013 available at http://www.otcair.org/upload/Documents/Formal%20Actions/Statement_EGUs.pdf

2. Ozone Transport Commission Draft Model Rule Control of Oil and Gas Fired Electric Generating Unit Boiler NOx Emissions, June 2010 available at http://www.otcair.org/upload/Documents/Meeting%20Materials/OTC%20Oil%20and%20Gas%20EGU%20Boiler%20NOx%20Model%20Rule%20Draft%20B_MOU_100603.pdf

3. Ozone Transport Commission Draft Model Rule Control of NOx Emissions from Natural Gas and Distillate Oil Fired HEDD Combustion Turbines, June 2010 available at http://www.otcair.org/upload/Documents/Model%20Rules/OTC%20Model%20Rule%20-%20HEDD%20Turbines%20Final.pdf

4. Ozone Transport Commission Memorandum of Understanding Among the States of the Ozone Transport Commission Concerning the Incorporation of High Electric Demand Day Emission Reduction Strategies into Ozone Attainment State Implementation Planning, March 2007, available at http://www.otcair.org/upload/Documents/Formal%20Actions/OTC_2007_SpecialMtg_%20HEDDMOU_Final_070302[1].pdf

5. OTC Modeling Domain – Revised 041213.pptx

6.  Ozone Transport Commission 2013 Annual Meeting, Stationary and Area Source Presentation, New Haven, Connecticut, slide 7-8, June 13, 2013

7. http://ampd.epa.gov/ampd/

8. http://www.nws.noaa.gov/climate/

9. “Final SAS Committee Update 040413 (2)”.pptx

10. OTC Domain HEDD, June 21-22, 2012.pptx

11. Coal SCR Scorecard 3. pptx

12.  Revised State Rules Summary Slides (CT, DE, NH, NJ, NY, & WI) 020414.pdf

13. NOx Rate Limit Refs.xlsx

14. Short Term NOx Limits Draft 9.xls

Ozone Transport Commission charge to the Stationary and Area Source Committee at November 2012 Fall meeting, Attached and  available at: http://www.otcair.org/upload/Documents/Formal%20Actions/Charge%20to%20SAS%20Committee.pdf

Ozone Transport Commission charge to the Stationary and Area Source Committee at November 2013

Fall meeting available at:

http://www.otcair.org/upload/Documents/Formal%20Actions/Chrg%20to%20SAS%20for%20Reg%20Attainment%20of%20Ozone.pdf

Ozone Transport Commission Draft Model Rule Control of Oil and Gas Fired Electric Generating Unit Boiler NOx Emissions, June 2010 available at

http://www.otcair.org/upload/Documents/Meeting%20Materials/OTC%20Oil%20and%20Gas%20EGU%20Boiler%20NOx%20Model%20Rule%20Draft%20B_MOU_100603.pdf

Ozone Transport Commission Draft Model Rule Control of NOx Emissions from Natural Gas and Distillate Oil Fired HEDD Turbines, June 2010 available at

http://www.otcair.org/upload/Documents/Model%20Rules/OTC%20Model%20Rule%20-%20HEDD%20Turbines%20Final.pdf

Ozone Transport Commission Memorandum of Understanding Among the States of the Ozone Transport Commission Concerning the Incorporation of High Electric Demand Day Emission Reduction Strategies into Ozone Attainment State Implementation Planning, March 2007, available at

http://www.otcair.org/upload/Documents/Formal%20Actions/OTC_2007_SpecialMtg_%20HEDDMOU_Final_070302[1].pdf

http://ampd.epa.gov/ampd/

http://www.nws.noaa.gov/climate/

Rates used in this section were provided by OTC States. The OTC EGU Subgroup requested from all OTC states short term rates that were on the books. The EGU subgroup received responses from Connecticut, Delaware, New Hampshire, New Jersey, and New York. The emission rate for Wisconsin was provided by an OTC state. The rates in this section are meant to be reflective of base load units, peaking units and units used with lower capacity may have other limits with which to comply.

Good emission control practices means operating the NOx emission controls as efficiently as possible in order to reduce NOx emissions as much as possible. Good emissions control would also include maintaining the emission controls according to manufacturer’s recommendations.